Sampling Rate Conversion: 16 kHz → 24 kHz

Analysis of Interpolation/Decimation Factor Error and MATLAB Implementation

# 1. Problem Statement

A sampling rate conversion system is designed to convert a 16 kHz signal to 24 kHz using an interpolation factor L and a decimation factor M. The original design specified L = 2 and M = 3, which produced an output sampling frequency of 10.67 kHz instead of the intended 24 kHz.

# 2. Root Cause Analysis

For a rational sample-rate converter, the output sampling frequency is related to the input by:

Fout = Fin × (L / M)

Substituting the original (incorrect) factors L = 2, M = 3:

Fout = 16000 × (2/3) = 10666.7 Hz  ✗

This confirms the reported 10.67 kHz output was arithmetically consistent with L = 2, M = 3 — but that ratio is wrong for the stated goal. To raise the sampling rate from 16 kHz to 24 kHz, the ratio L/M must equal Fout/Fin = 24000/16000 = 3/2, which requires the interpolation factor to be larger than the decimation factor, not smaller.

- Error: The interpolation factor (L) and decimation factor (M) were swapped.

- L = 2, M = 3 actually implements the reverse conversion (24 kHz → 16 kHz), not 16 kHz → 24 kHz.

- Correct factors: L = 3 (interpolation), M = 2 (decimation), giving Fout = 16000 × 3/2 = 24000 Hz.

# 3. Corrected Calculation

| Parameter | Value |
| --- | --- |
| Input sampling frequency (Fin) | 16000 Hz |
| Target output sampling frequency | 24000 Hz |
| Erroneous factors | L = 2, M = 3 |
| Erroneous output (Fin × L/M) | 10666.7 Hz |
| Corrected factors | L = 3, M = 2 |
| Corrected output (Fin × L/M) | 24000 Hz |

# 4. Implementation Approach

The corrected system was implemented in MATLAB as an explicit two-stage cascade:

- Stage 1 — Interpolation by L = 3: insert (L−1) zeros between input samples, then apply an anti-imaging FIR lowpass filter (gain L to compensate for zero-insertion loss).

- Stage 2 — Decimation by M = 2: apply an anti-aliasing FIR lowpass filter at the intermediate rate (Fin × L = 48000 Hz), then discard every Mth sample.

- Cutoff frequency for both filters set to min(Fin, Fout)/2 = 8000 Hz to prevent both imaging and aliasing.

- Result cross-checked against MATLAB's built-in resample(x, L, M) function for verification.

# 5. MATLAB Code (Corrected)

%% Sampling Rate Conversion: 16 kHz -> 24 kHz % CORRECTED VERSION % % ERROR IN ORIGINAL DESIGN: % Original used L = 2 (interpolation), M = 3 (decimation) % Fout = Fin * L/M = 16000 * 2/3 = 10666.7 Hz (WRONG, and this is % actually the ratio for 24 kHz -> 16 kHz, i.e. L and M were swapped) % % CORRECT DESIGN: % Need Fout/Fin = 24000/16000 = 3/2 => L = 3, M = 2 % Fout = Fin * L/M = 16000 * 3/2 = 24000 Hz (CORRECT) clear; clc; close all; %% Parameters Fin = 16000; % Input sampling frequency (Hz) L = 3; % Interpolation factor (CORRECTED, was 2) M = 2; % Decimation factor (CORRECTED, was 3) Fout_correct = Fin * L / M; Fout_wrong = Fin * 2 / 3; % what the erroneous design produced fprintf('--- Frequency Check ---\n'); fprintf('Erroneous design (L=2, M=3): Fout = %.2f Hz\n', Fout_wrong); fprintf('Corrected design (L=%d, M=%d): Fout = %.2f Hz\n', L, M, Fout_correct); %% Generate a test signal (sum of two tones) at Fin = 16 kHz Fs_in = Fin; t_in = (0:1/Fs_in:0.01-1/Fs_in)'; % 10 ms duration f1 = 1000; f2 = 3000; % test tones (Hz) x = sin(2*pi*f1*t_in) + 0.5*sin(2*pi*f2*t_in); %% ---- STEP 1: Interpolation by L = 3 ---- % Upsample by inserting (L-1) zeros between samples, then lowpass filter x_up = zeros(L*length(x), 1); x_up(1:L:end) = x; % Anti-imaging lowpass filter, cutoff at min(Fin,Fout)/2 relative to % the upsampled rate (Fs_in*L) Fs_up = Fs_in * L; % intermediate rate = 48000 Hz cutoff = min(Fs_in, Fout_correct)/2; % = 8000 Hz Nf = 64; % FIR filter order h_interp = L * fir1(Nf, cutoff/(Fs_up/2)); % gain L compensates zero-insertion loss x_interp = filter(h_interp, 1, x_up); %% ---- STEP 2: Decimation by M = 2 ---- % Anti-aliasing lowpass filter at the upsampled rate, then downsample h_decim = fir1(Nf, cutoff/(Fs_up/2)); x_filt = filter(h_decim, 1, x_interp); y = x_filt(1:M:end); % keep every Mth sample Fs_out = Fs_up / M; % 48000/2 = 24000 Hz fprintf('\n--- Verification ---\n'); fprintf('Input length : %d samples at %d Hz (%.4f s)\n', length(x), Fs_in, length(x)/Fs_in); fprintf('Output length: %d samples at %d Hz (%.4f s)\n', length(y), Fs_out, length(y)/Fs_out); fprintf('Achieved output sampling frequency = %d Hz\n', Fs_out); %% ---- Cross-check using MATLAB's built-in resample/upfirdn ---- y_check = resample(x, L, M); fprintf('\nBuilt-in resample(x,3,2) length = %d samples => Fs_out = %.0f Hz\n', ... length(y_check), Fs_in*L/M); %% ---- Plot: time domain comparison ---- figure('Name','Sampling Rate Conversion 16kHz -> 24kHz'); subplot(2,1,1); stem(t_in(1:60), x(1:60), 'b', 'filled'); title('Input Signal @ 16 kHz'); xlabel('Time (s)'); ylabel('Amplitude'); grid on; t_out = (0:length(y)-1)/Fs_out; subplot(2,1,2); stem(t_out(1:90), y(1:90), 'r', 'filled'); title(sprintf('Output Signal @ %d Hz (L=%d, M=%d)', Fs_out, L, M)); xlabel('Time (s)'); ylabel('Amplitude'); grid on; %% ---- Plot: frequency domain comparison ---- figure('Name','Spectrum Comparison'); Nfft = 2048; X = abs(fft(x, Nfft)); f_in_axis = (0:Nfft-1)*(Fs_in/Nfft); subplot(2,1,1); plot(f_in_axis(1:Nfft/2), X(1:Nfft/2)); title('Input Spectrum (16 kHz)'); xlabel('Frequency (Hz)'); ylabel('¦X(f)¦'); grid on; Y = abs(fft(y, Nfft)); f_out_axis = (0:Nfft-1)*(Fs_out/Nfft); subplot(2,1,2); plot(f_out_axis(1:Nfft/2), Y(1:Nfft/2)); title(sprintf('Output Spectrum (%d Hz)', Fs_out)); xlabel('Frequency (Hz)'); ylabel('¦Y(f)¦'); grid on;

# 6. Simulation Results

## 6.1 Time-Domain Signals

The input signal at 16 kHz (160 samples, two tones at 1 kHz and 3 kHz) is converted to a 240-sample output at 24 kHz. The waveform shape and periodicity are preserved, confirming a correct, distortion-free rate change.

Figure 1. Input signal @ 16 kHz (top) vs. output signal @ 24000 Hz, L = 3, M = 2 (bottom).

## 6.2 Frequency-Domain Spectra

The spectrum of the output signal preserves both tones at 1 kHz and 3 kHz with no visible aliasing or imaging artifacts across the extended 0–12 kHz range, confirming the anti-imaging/anti-aliasing filtering was effective.

Figure 2. Input spectrum @ 16 kHz (top) vs. output spectrum @ 24000 Hz (bottom), tones preserved at 1 kHz and 3 kHz.

# 7. Verification Output

--- Frequency Check --- Erroneous design (L=2, M=3): Fout = 10666.67 Hz Corrected design (L=3, M=2): Fout = 24000.00 Hz --- Verification --- Input length : 160 samples at 16000 Hz (0.0100 s) Output length: 240 samples at 24000 Hz (0.0100 s) Achieved output sampling frequency = 24000 Hz Built-in resample(x,3,2) length = 240 samples => Fs_out = 24000 Hz

# 8. Conclusion

The original design error was a swap of the interpolation and decimation factors: L = 2, M = 3 yields Fin × 2/3 = 10.67 kHz, which is the correct ratio for downsampling 24 kHz to 16 kHz, not for upsampling 16 kHz to 24 kHz. Correcting the factors to L = 3, M = 2 yields the intended output rate:

Fout = 16000 × (3/2) = 24000 Hz  ✓

This was confirmed both analytically and by MATLAB simulation (explicit multirate implementation and the built-in resample function), with time- and frequency-domain plots showing clean, artifact-free conversion.