| Expt 8 | Multirate Signal Processing |
| --- | --- |
| Date: | 8.1 Implementation and Analysis of Decimation and Interpolation in Multirate Signal Processing Using MATLAB |

## Objectives

- To generate a discrete-time sinusoidal sequence for multirate signal processing.

- To perform decimation of a discrete-time sequence by a factor of 2.

- To perform Interpolation of a discrete-time sequence by a factor of 2.

- To analyze and compare the original, decimated, and interpolated sequences.

## Procedure

- Open MATLAB and initialize the workspace using clc, clear, and close all.

- Generate a discrete-time sinusoidal sequence with the desired frequency and sampling rate.

- Perform decimation by selecting every 𝑀𝑡ℎsample using the downsample() function.

- Perform interpolation by inserting zeros and filtering using the upsample() function.

- Display the original signal samples.

- Display the decimated signal samples.

- Display the interpolated signal samples.

- Plot and compare the original, decimated, and interpolated signals.

## Program (Matlab Code)

clc; clear;close all;

% Original Sequence n= 0:40;

x = sin(0.3*pi*n);

% Decimation by 2 y= x(1:2:end);

nd = 0:length(y)-1;

% Interpolation by 2

v = zeros(1,2*length(x)); v(1:2:end)= x;

ni = 0:length(v)-1;

% Figure 1: Original Sequence figure;

stem(n,x,'filled'); grid on;

title('Original Sequence x[n]'); xlabel('n'); ylabel('Amplitude');

% Figure 2: Decimated Sequence figure;

stem(nd,y,'filled'); grid on;

title('Decimated Sequence by 2'); xlabel('n');

ylabel('Amplitude');

% Figure 3: Interpolated Sequence

figure; stem(ni,v,'filled');grid on;

title('Interpolated Sequence by 2'); xlabel('n');

ylabel('Amplitude');

% Figure 4: Comparison figure;

stem(n,x,'b'); hold on; stem(0:2:length(v)-2,v(1:2:end),'r');

legend('Original x[n]','Interpolated Samples'); grid on;

title('Comparison of Sequences'); xlabel('n');

ylabel('Amplitude'); Output: Objective 1:

# Objective 2:

# Objective 3:

# Objective 4:

## Result :

- A discrete-time sinusoidal sequence was successfully generated for multirate processing.

- The discrete-time sequence was successfully decimated by a factor of 2, reducing the number of samples and the sampling rate.

- The discrete-time sequence was successfully interpolated by a factor of 2, increasing the number of samples and the sampling rate.

- . The effects of downsampling and upsampling were successfully analyzed by comparing the original, decimated, and interpolated signals.

| Expt 8 | Multirate Signal Processing |
| --- | --- |
| Date: | 8.2 Polyphase Implementation of Decimation and Interpolation Filters Using MATLAB |

## Objectives:

- To perform sampling rate reduction of a discrete-time signal using a polyphase FIR decimation filter with a decimation factor of 2.

- To perform sampling rate increase of a discrete-time signal using a polyphase FIR interpolation filter with an interpolation factor of 2.

- To reconstruct the original signal from the decimated signal through interpolation and compare the reconstructed signal with the original input signal.

- To demonstrate the computational efficiency and effectiveness of polyphase filter structures in multirate digital signal processing applications.

## Procedure

- Generate the Input Signal

- Design the FIR Low-Pass Filter

- Perform Polyphase Decimation Choose the decimation factor 𝑴 = 𝟐.

- Perform Polyphase Interpolation Choose the interpolation factor 𝐿 = 2.

- Analyze the Signals Plot the original signal, decimated signal, and interpolated signal in the time domain.

- Analyze the Frequency Response Obtain the frequency response of the FIR filter using the freqz() function.

- Verify Signal Lengths Display the lengths of the original, decimated, and interpolated signals.

- Verify successful decimation by a factor of 2. Verify successful interpolation by a factor of

- Conclude that polyphase implementation achieves efficient multirate processing with reduced computational complexity.

## Program (Matlab coding):

clc; clear; close all;

%% Input Signal

Fs = 1000;	% Sampling Frequency

t = 0:1/Fs:1;	% Time Vector

% Test Signal

x = sin(2*pi*50*t) + 0.5*sin(2*pi*120*t);

%% FIR Low-Pass Filter Design

N = 20;	% Filter Order

Fc = 0.4;	% Normalized Cutoff Frequency h = fir1(N,Fc,hamming(N+1));

%% Decimation Factor M = 2;

% Polyphase Decimation y_dec =upfirdn(x,h,1,M);

%% Interpolation Factor L = 2;

% Polyphase Interpolation y_int = upfirdn(y_dec,h,L,1);

%% Adjust Length for Comparison if length(y_int) > length(x)

y_int = y_int(1:length(x)); elseif length(y_int) < length(x)

x = x(1:length(y_int)); end

%% Time Axes

t1 = (0:length(x)-1)/Fs;

t2 = (0:length(y_dec)-1)/(Fs/M); t3 = (0:length(y_int)-1)/Fs;

%% BEST GRAPH OUTPUT

figure('Name','Polyphase Decimation and Interpolation','NumberTitle','off');

% Original Signal subplot(4,1,1) plot(t1,x,'b','LineWidth',1.5) title('Original Input Signal') xlabel('Time (s)')

ylabel('Amplitude') grid on

% Decimated Signal subplot(4,1,2) stem(t2,y_dec,'r','filled') title('Decimated Signal (M = 2)') xlabel('Time (s)') ylabel('Amplitude')

grid on

% Interpolated Signal subplot(4,1,3) plot(t3,y_int,'k','LineWidth',1.5)

title('Interpolated/Reconstructed Signal (L = 2)') xlabel('Time (s)')

ylabel('Amplitude') grid on

% Comparison subplot(4,1,4) plot(t1,x,'b','LineWidth',1.5) hold on

plot(t3,y_int,'r--','LineWidth',1.5)

title('Comparison of Original and Reconstructed Signals') xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal') grid on

%% Display Information

disp('	');

disp('POLYPHASE DECIMATION & INTERPOLATION');

disp('	');

fprintf('Original Signal Length	= %d\n',length(x)); fprintf('Decimated Signal Length		= %d\n',length(y_dec)); fprintf('Interpolated Signal Length = %d\n',length(y_int));

%% Separate Figure for Better Visualization figure;

plot(t1,x,'b','LineWidth',2) hold on

plot(t3,y_int,'r--','LineWidth',2) title('Original vs Reconstructed Signal') xlabel('Time (s)')

ylabel('Amplitude')

legend('Original Signal','Reconstructed Signal') grid on

## Output:

POLYPHASE DECIMATION & INTERPOLATION

Original Signal Length = 1001 Decimated Signal Length = 511 Interpolated Signal Length = 1001 Result :

- The input signal was successfully decimated by a factor of 2 using a polyphase FIR decimation filter.

- The input signal was successfully interpolated by a factor of 2 using a polyphase FIR interpolation filter.

- The original signal was reconstructed from the decimated signal and closely matched the input signal.

- The polyphase filter structure reduced computational complexity and efficiently performed multirate signal processing operations.