| Expt 4 | Design of Low Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 4..1. Design and Analysis of an FIR Low-Pass Filter Using the Rectangular Window Technique in MATLAB |

  - Open MATLAB and create a new script file.

  - Initialize the workspace using clc, clear, and close all.

  - Specify the filter design parameters: sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR Low-Pass Filter using the fir1() function with a Rectangular Window (rectwin).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude and phase responses using the freqz() function.

  - Plot and analyze the impulse response of the designed filter.

  - Generate the pole-zero plot and examine the filter characteristics.

  - Verify the stability and frequency-selective performance of the FIR Low-Pass Filter.

## Program(Matlab Code)

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	% Passband cutoff (Hz)

fs = 4000;	% Sampling frequency (Hz)

N = 50;	% Filter order (even number preferred)

wc = fp/(fs/2);  % Normalized cutoff frequency (0 to 1)

%% --- 1. FIR Low Pass using Rectangular Window ---b_rect = fir1(N, wc, 'low', rectwin(N+1));

disp('--- Rectangular Window Coefficients ---'); disp(b_rect');

%% --- 2. FIR Low Pass using Hamming Window ---b_hamm = fir1(N, wc, 'low', hamming(N+1));

disp('--- Hamming Window Coefficients ---');disp(b_hamm');

%% --- 3. FIR Low Pass using Hanning Window ---b_hann = fir1(N, wc, 'low', hanning(N+1));

disp('--- Hanning Window Coefficients ---');disp(b_hann');

%% --- Plot Frequency Responses ---figure;

freqz(b_rect,1);

title('Low-pass FIR using Rectangular Window'); figure;

freqz(b_hamm,1);

title('Low-pass FIR using Hamming Window'); figure;

freqz(b_hann,1);

title('Low-pass FIR using Hanning Window'); OUTPUT;

## Objective 1:

--- Rectangular Window Coefficients ---0.0126

-0.0000

-0.0137

0.0000

0.0150

-0.0000

-0.0166

0.0000

0.0185

-0.0000

-0.0210

0.0000

0.0242

-0.0000

-0.0286

0.0000

0.0349

-0.0000

-0.0449

0.0000

0.0629

-0.0000

-0.1048

0.0000

0.3145

0.4940

0.3145

0.0000

-0.1048

-0.0000

0.0629

0.0000

-0.0449

-0.0000

0.0349

0.0000

-0.0286

-0.0000

0.0242

0.0000

-0.0210

-0.0000

0.0185

0.0000

-0.0166

-0.0000

0.0150

0.0000

-0.0137

-0.0000

0.0126

## Objective 2:

## Result

- An FIR Low-Pass Filter was successfully designed using the Rectangular Window technique for the specified cutoff frequency, sampling frequency, and filter order.

- The FIR filter coefficients were successfully generated using MATLAB and the designed filter was implemented.

- The magnitude response and phase response of the designed FIR Low-Pass Filter were obtained and analyzed. The magnitude response exhibited low-pass characteristics, while the phase response demonstrated the expected linear-phase behavior.

| Expt 4 | Design of Low Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 4.2. Design and Analysis of an FIR Low-Pass Filter Using the Hamming Window Technique in MATLAB |

Procedure

- To design an FIR Low-Pass Filter using the Hamming Window technique for the given cutoff frequency, sampling frequency, and filter order specifications.

- To generate the FIR filter coefficients using the Hamming Window method in MATLAB and implement the designed filter.

- To obtain and analyze the magnitude response and phase response of the FIR Low-Pass Filter designed using the Hamming Window.

  - Open MATLAB and create a new script file.

  - Clear the workspace, command window, and close all figure windows.

  - Specify the sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR Low-Pass Filter using the fir1() function with the Hamming Window (hamming).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude and phase responses of the designed filter using the freqz() function.

  - Plot and analyze the frequency response characteristics of the filter.

  - Verify the low-pass filtering behavior by observing the passband and stopband responses.

  - Record the observations and conclude the performance of the FIR Low-Pass Filter designed using the Hamming Window technique.

## Program (Matlab Code)

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	% Passband cutoff (Hz)

fs = 4000;	% Sampling frequency (Hz)

N = 50;	% Filter order (even number preferred) wc = fp/(fs/2); % Normalized cutoff frequency (0 to 1)

%% --- FIR Low Pass using Hamming Window ---

b_hamm = fir1(N, wc, 'low', hamming(N+1)); disp('--- Hamming Window Coefficients ---'); disp(b_hamm');

%% --- Plot Frequency Responses ---figure;

freqz(b_hamm,1);

title('Low-pass FIR using Hamming Window'); Output

--- Hamming Window Coefficients ---0.0010

-0.0000

-0.0013

0.0000

0.0021

-0.0000

-0.0034

0.0000

0.0055

-0.0000

-0.0084

0.0000

0.0125

-0.0000

-0.0181

0.0000

0.0260

-0.0000

-0.0379

0.0000

0.0580

-0.0000

-0.1026

0.0000

0.3168

0.4995

0.3168

0.0000

-0.1026

-0.0000

0.0580

0.0000

-0.0379

-0.0000

0.0260

0.0000

-0.0181

-0.0000

0.0125

0.0000

-0.0084

-0.0000

0.0055

0.0000

-0.0034

-0.0000

0.0021

0.0000

-0.0013

-0.0000

0.0010

## Result

- The FIR Low-Pass Filter was successfully designed using the Hamming Window technique for the specified filter parameters.

- The FIR filter coefficients were generated and implemented successfully in MATLAB.

- The magnitude and phase responses of the designed filter were obtained and analyzed.

- The filter exhibited the desired low-pass characteristics with improved stopband attenuation compared to the Rectangular Window method.

| Expt 4 | Design of Low Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 4.3. Design and Analysis of an FIR Low-Pass Filter Using the Hanning Window Technique in MATLAB |

Procedure

- To design an FIR Low-Pass Filter using the Hanning Window technique for the given cutoff frequency, sampling frequency, and filter order specifications.

- To generate the FIR filter coefficients using the Hanning Window method in MATLAB and implement the designed filter.

- To obtain and analyze the magnitude response and phase response of the FIR Low-Pass Filter designed using the Hanning Window.

  - Open MATLAB and create a new script file.

  - Clear the workspace, command window, and close all figure windows.

  - Specify the sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR Low-Pass Filter using the fir1() function with the Hanning (Hann) Window (hanning).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude response and phase response of the designed filter using the freqz() function.

  - Plot and analyze the frequency response characteristics of the filter.

  - Verify the low-pass filtering behavior by observing the passband and stopband regions.

  - Record the observations and conclude the performance of the FIR Low-Pass Filter designed using the Hanning Window technique.

## Program (Matlab Code):

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	% Passband cutoff (Hz)

fs = 4000;	% Sampling frequency (Hz)

N = 50;	% Filter order (even number preferred) wc = fp/(fs/2); % Normalized cutoff frequency (0 to 1)

%% --- FIR Low Pass using Hanning Window ---b_hann = fir1(N, wc, 'low', hanning(N+1));

disp('--- Hanning Window Coefficients ---');disp(b_hann');

%% --- Plot Frequency Responses ---figure;

freqz(b_hann,1);

title('Low-pass FIR using Hanning Window'); OUTPUT;

--- Hanning Window Coefficients ---0.0000

-0.0000

-0.0004

0.0000

0.0013

-0.0000

-0.0028

0.0000

0.0050

-0.0000

-0.0081

0.0000

0.0122

-0.0000

-0.0179

0.0000

0.0259

-0.0000

-0.0378

0.0000

0.0580

-0.0000

-0.1027

0.0000

0.3172

0.5000

0.3172

0.0000

-0.1027

-0.0000

0.0580

0.0000

-0.0378

-0.0000

0.0259

0.0000

-0.0179

-0.0000

0.0122

0.0000

-0.0081

-0.0000

0.0050

0.0000

-0.0028

-0.0000

0.0013

0.0000

-0.0004

-0.0000

0.0000

## Result

- The FIR Low-Pass Filter was successfully designed using the Hanning Window technique for the specified filter parameters.

- The FIR filter coefficients were generated and implemented successfully in MATLAB.

- The magnitude and phase responses of the designed filter were obtained and analyzed.

- The filter exhibited the desired low-pass characteristics with reduced sidelobes and improved stopband attenuation compared to the Rectangular Window method.