| Expt 12 | FIR Filters using DSP Processor |
| --- | --- |
| Date: | 12.1 Design and Implementation of FIR High Pass filter using Rectangular Window |

## Objectives:

- To design an FIR filter using the Rectangular Window method with specified filter requirements.

- To implement the designed FIR filter on the TMS320C5416 DSP Starter Kit.

- To apply an input signal to the FIR filter and obtain the filtered output using the DSP processor.

- To analyze and verify the performance of the FIR filter by observing the input and output responses on the TMS320C5416 Starter Kit.

## Hardware and Software Required:

DSP – TMS320VC5416-5416 Board

Cpu

ME Universal Debugger DSP - 5416

## Procedure:

- Connect the serial port cable between the PC and the VSK-5416 kit.

- Switch ON the VSK-5416 kit and press the RESET button.

- Open Vi Universal Debugger.

- Select Serial → Port Settings → Auto Detect and verify that the message "Vi Universal Debugger found in COMx" appears.

- Create a new project (File → New Project) and assign a project name.

- Create a new source file (File → New File) and write the program.

- Save the source file.

- Add the source file to the project.

- Add the command file (Micro5416.cmd) to the project.

- Save the project.

- Build/Rebuild the project and ensure there are no errors.

- Download the executable file to the VSK-5416 kit.

- Run the program.

- Observe and verify the output.

;**************************************************************************

## HIGH PASS FIR FILTER

;**************************************************************************

;Sampling freq	: 43khz

;Cut-off freq	: 2khz

;N	52

;Filter type	: High pass filter

;Window type	: Rectangular

;Program Description:

;1. Make all the x(n) zero initially

;2. Read the data from the adc.

;3. Store the adc data in x(0)

;4. Make the pointer to point the x(n_end)

;5. Perform the convolution of x(n) and the coefficients h(n) using

; MACD instruction.

;6. Send the convolution output to the dac

;7. Repeat from step 2.

.mmregs

.text START:

| STM | #01h,ST0 | ;initialize the data page pointer |
| --- | --- | --- |
| RSBX | CPL | ;Make the processor to work using DP |
| RSBX | FRCT | ;reset the fractional mode bit |
| NOP |  |  |
| NOP |  |  |

;*****loop to make all x(n) zero initially*****

STM	#150H,AR1	;initialize ar1 to point to x(n) LD	#0H,A	;make acc zero

RPT	#34H

STL	A,*AR1+	;make all x(n) zero

;*****to read the adc data and store it in x(0)***** LOOP:

PORTR	06,0	;start of conversion CHK_BUSY:

| ; PORTR ; BITF ; BC | 07,0 0,#20H CHK_BUSY,TC | ;check for busy |
| --- | --- | --- |
| PORTR | 04,0 | ;read the adc data |
| LD | 0,A |  |
| AND | #0FFFH,A | ;AND adc data with 0fffh for 12 bit adc |
| XOR | #0800H,A | ;recorrect the 2's complement adc data |
| SUB | #800H,A | ;remove the dc shift |
| STM | #150H,AR1 | ;initialize ar1 with x(0) |
| STL | A,*AR1 | ;store adc data in x(0) |

TM	#183H,AR2	;initialize ar2 with x(n_end)

;*****start of convolution*****

| LD | #0H,A | ;sum is 0 initially |
| --- | --- | --- |
| RPT | #33H |  |
| MACD | *AR2-,TABLE,A | ;convolution process |
| STH | A,1,0H |  |
| LD | 0H,A |  |
| ADD | #800H,A | ;add the dc shift to the convolution output |
| STL | A,1H |  |
| PORTW | 1H,04H | ;send the output to the dac |
| B | LOOP |  |

TABLE:

| .word | 0FCEFH |
| --- | --- |

| .word | 62H |
| --- | --- |
| .word | 0FD50H |
| .word | 14AH |
| .word | 0FE1BH |
| .word | 28FH |
| .word | 0FF11H |
| .word | 3E5H |
| .word | 0FFD1H |
| .word | 4ECH |
| .word | 0FFF5H |
| .word | 54FH |
| .word | 0FF28H |
| .word | 4DAH |
| .word | 0FD38H |
| .word | 398H |
| .word | 0FA2EH |
| .word | 1DDH |
| .word | 0F627H |
| .word | 55H |
| .word | 0F131H |
| .word | 4BH |
| .word | 0EA6DH |
| .word | 568H |
| .word | 0D950H |
| .word | 459EH |
| .word | 459EH |
| .word | 0D950H |
| .word | 568H |

| .word | 0EA6DH |
| --- | --- |
| .word | 4BH |
| .word | 0F131H |
| .word | 55H |
| .word | 0F627H |
| .word | 1DDH |
| .word | 0FA2EH |
| .word | 398H |
| .word | 0FD38H |
| .word | 4DAH |
| .word | 0FF28H |
| .word | 54FH |
| .word | 0FFF5H |
| .word | 4ECH |
| .word | 0FFD1H |
| .word | 3E5H |
| .word | 0FF11H |
| .word | 28FH |
| .word | 0FE1BH |
| .word | 14AH |
| .word | 0FD50H |
| .word | 62H |
| .word | 0FCEFH |

## Output:

## Result:

- The FIR filter was successfully designed using the Rectangular Window method.

- The designed filter was implemented and executed on the TMS320C5416 DSP Starter Kit.

- The input signal was processed through the FIR filter, and the filtered output was obtained.

- The output response verified the desired filtering operation, demonstrating the successful implementation of the FIR filter on the DSP processor.