| Expt 5 | Design of High Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 5.1 Generate the coefficients of an FIR High-Pass filter using Rectangular windowing techniques. |

## Objectiv:

## 1.To design an FIR High-Pass Filter using the Hamming Window technique for Given cutoff frequency, sampling frequency, and filter order specifications.

## To generate the FIR filter coefficients using the Hamming Window method I,MATLAB and implement the designed filter.

## To obtain and analyze the magnitude response and phase response of the FIR High-Pass Filter designed using the Hamming Window.

Procedure

- To design an FIR High-Pass Filter using the Rectangular Window technique for the given cutoff frequency, sampling frequency, and filter order specifications.

- To generate the FIR filter coefficients using the Rectangular Window method in MATLAB and implement the designed filter.

- To obtain and analyze the magnitude response and phase response of the FIR High-Pass Filter designed using the Rectangular Window.

  - Open MATLAB and create a new script file.

  - Clear the workspace, command window, and close all figure windows.

  - Specify the sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR High-Pass Filter using the fir1() function with the 'high' option and a Rectangular Window (rectwin).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude response and phase response of the designed filter using the freqz() function.

  - Plot and analyze the frequency response characteristics of the filter.

  - Verify the high-pass filtering behavior by observing the attenuation of low-frequency components and the passage of high-frequency components.

  - Record the observations and conclude the performance of the FIR High-Pass Filter designed using the Rectangular Window technique.

## Program (Matlab Code):

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	 % High-pass cutoff frequency (Hz) fs = 8000;	% Sampling frequency (Hz)

N = 50;	% Filter order

wc = fp/(fs/2);  % Normalized cutoff (0–1)

%% --- 1. FIR High Pass using Rectangular Window ---b_rect = fir1(N, wc, 'high', rectwin(N+1));

disp('--- High-Pass Coefficients (Rectangular Window) ---'); disp(b_rect');

%% --- 2. FIR High Pass using Hamming Window ---b_hamm = fir1(N, wc, 'high', hamming(N+1));

disp('--- High-Pass Coefficients (Hamming Window) ---'); disp(b_hamm');

%% --- 3. FIR High Pass using Hanning Window ---b_hann = fir1(N, wc, 'high', hanning(N+1));

disp('--- High-Pass Coefficients (Hanning Window) ---'); disp(b_hann');

%% --- Frequency Response Plots ---figure;

freqz(b_rect,1);

title('High-pass FIR using Rectangular Window'); figure;

freqz(b_hamm,1);

title('High-pass FIR using Hamming Window'); figure;

freqz(b_hann,1);

title('High-pass FIR using Hanning Window');

OUTPUT:

--- High-Pass Coefficients (Rectangular Window) ---

-0.0089

-0.0000

0.0097

0.0143

0.0106

-0.0000

-0.0117

-0.0175

-0.0131

-0.0000

0.0148

0.0225

0.0171

-0.0000

-0.0202

-0.0314

-0.0247

-0.0000

0.0318

0.0524

0.0445

-0.0000

-0.0741

-0.1572

-0.2223

0.7407

-0.2223

-0.1572

-0.0741

-0.0000

0.0445

0.0524

0.0318

-0.0000

-0.0247

-0.0314

-0.0202

-0.0000

0.0171

0.0225

0.0148

-0.0000

-0.0131

-0.0175

-0.0117

-0.0000

0.0106

0.0143

0.0097

-0.0000

-0.0089

## Result

- The FIR High-Pass Filter was successfully designed using the Rectangular Window technique for the specified cutoff frequency, sampling frequency, and filter order.

- The FIR filter coefficients were successfully generated using MATLAB and the designed filter was implemented.

- The magnitude response and phase response of the designed FIR High-Pass Filter were obtained and analyzed. The magnitude response showed attenuation of low-frequency components and passage of high-frequency components, confirming the desired high-pass

filtering behavior. The phase response exhibited the expected linear-phase characteristics of an FIR filter.

| Expt 5 | Design of High Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 5.2. Generate the coefficients of an FIR High-Pass filter using Hamming windowing techniques |

Procedure

- To design an FIR High-Pass Filter using the Hamming Window technique for the given cutoff frequency, sampling frequency, and filter order specifications.

- To generate the FIR filter coefficients using the Hamming Window method in MATLAB and implement the designed filter.

- To obtain and analyze the magnitude response and phase response of the FIR High-Pass Filter designed using the Hamming Window.

  - Open MATLAB and create a new script file.

  - Clear the workspace, command window, and close all figure windows.

  - Specify the sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR High-Pass Filter using the fir1() function with the 'high' option and a Hamming Window (hamming).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude response and phase response of the designed filter using the freqz() function.

  - Plot and analyze the frequency response characteristics of the filter.

  - Verify the high-pass filtering behavior by observing the attenuation of low-frequency components and the passage of high-frequency components.

  - Record the observations and conclude the performance of the FIR High-Pass Filter designed using the Hamming Window technique.

## Program (Matlab Code):

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	 % High-pass cutoff frequency (Hz) fs = 8000;	% Sampling frequency (Hz)

N = 50;	% Filter order

wc = fp/(fs/2);  % Normalized cutoff (0–1)

%% --- FIR High Pass using Hamming Window ---b_hamm = fir1(N, wc, 'high', hamming(N+1));

disp('--- High-Pass Coefficients (Hamming Window) ---'); disp(b_hamm');

%% --- Frequency Response Plots ---figure;

freqz(b_hamm,1);

title('High-pass FIR using Hamming Window'); title('High-pass FIR using Hanning Window');

--- High-Pass Coefficients (Hamming Window) ---

-0.0007

-0.0000

0.0009

0.0016

0.0015

-0.0000

-0.0024

-0.0044

-0.0039

-0.0000

0.0060

0.0103

0.0088

-0.0000

-0.0128

-0.0217

-0.0184

-0.0000

0.0268

0.0464

0.0410

-0.0000

-0.0725

-0.1567

-0.2240

0.7493

-0.2240

-0.1567

-0.0725

-0.0000

0.0410

0.0464

0.0268

-0.0000

-0.0184

-0.0217

-0.0128

-0.0000

0.0088

0.0103

0.0060

-0.0000

-0.0039

-0.0044

-0.0024

-0.0000

0.0015

0.0016

0.0009

-0.0000

-0.0007

## Result:

- The FIR High-Pass Filter was successfully designed using the Hamming Window technique for the specified cutoff frequency, sampling frequency, and filter order.

- The FIR filter coefficients were successfully generated using the Hamming Window method in MATLAB and the filter was implemented.

- The magnitude response and phase response of the designed FIR High-Pass Filter were obtained and analyzed. The magnitude response showed effective attenuation of low-frequency components and passage of high-frequency components, confirming the desired high-pass filtering behavior. The phase response exhibited the expected linear-phase characteristics of an FIR filter.

| Expt 5 | Design of High Pass Filter using Windowing Techniques |
| --- | --- |
| Date: | 5.3. Generate the coefficients of an FIR High-Pass filter using Hanning windowing techniques |

Procedure:

- To design an FIR High-Pass Filter using the Hanning Window technique for the given cutoff frequency, sampling frequency, and filter order specifications.

- To generate the FIR filter coefficients using the Hanning Window method in MATLAB and implement the designed filter.

- To obtain and analyze the magnitude response and phase response of the FIR High-Pass Filter designed using the Hanning Window.

  - Open MATLAB and create a new script file.

  - Clear the workspace, command window, and close all figure windows.

  - Specify the sampling frequency, cutoff frequency, and filter order.

  - Calculate the normalized cutoff frequency.

  - Design the FIR High-Pass Filter using the fir1() function with the 'high' option and a Hanning (Hann) Window (hanning).

  - Generate and display the FIR filter coefficients.

  - Obtain the magnitude response and phase response of the designed filter using the freqz() function.

  - Plot and analyze the frequency response characteristics of the filter.

  - Verify the high-pass filtering behavior by observing the attenuation of low-frequency components and the passage of high-frequency components.

  - Record the observations and conclude the performance of the FIR High-Pass Filter designed using the Hanning Window technique.

## Program (Matlab Code)

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000;	 % High-pass cutoff frequency (Hz) fs = 8000;	% Sampling frequency (Hz)

N = 50;	% Filter order

wc = fp/(fs/2);  % Normalized cutoff (0–1)

%% --- FIR High Pass using Hanning Window ---

b_hann = fir1(N, wc, 'high', hanning(N+1));

disp('--- High-Pass Coefficients (Hanning Window) ---'); disp(b_hann');

%% --- Frequency Response Plots ---figure;

freqz(b_hann,1);

title('High-pass FIR using Hanning Window'); Output:

--- High-Pass Coefficients (Hanning Window) ---

-0.0000

-0.0000

0.0003

0.0008

0.0009

-0.0000

-0.0020

-0.0038

-0.0035

-0.0000

0.0057

0.0100

0.0087

-0.0000

-0.0127

-0.0216

-0.0183

-0.0000

0.0267

0.0464

0.0410

-0.0000

-0.0726

-0.1568

-0.2243

0.7500

-0.2243

-0.1568

-0.0726

-0.0000

0.0410

0.0464

0.0267

-0.0000

-0.0183

-0.0216

-0.0127

-0.0000

0.0087

0.0100

0.0057

-0.0000

-0.0035

-0.0038

-0.0020

-0.0000

0.0009

0.0008

0.0003

-0.0000

-0.0000

>>

## Result

- The FIR High-Pass Filter was successfully designed using the Hanning Window technique for the specified cutoff frequency, sampling frequency, and filter order.

- The FIR filter coefficients were successfully generated using MATLAB and the designed filter was implemented.

- The magnitude response and phase response of the designed FIR High-Pass Filter were obtained and analyzed. The magnitude response exhibited attenuation of low-frequency components and passage of high-frequency components, confirming the desired high-pass filtering characteristics. The phase response demonstrated the expected linear-phase behavior of the FIR filter.