| Expt 3 | Performance Analysis of IIR Filters |
| --- | --- |
| Date | 3.1 Design and Comparative Analysis of Butterworth and Chebyshev Type-I IIR Band-Pass Filters Using MATLAB |

## Objectives

- To design Butterworth and Chebyshev Type-I IIR Band-Pass Filters in MATLAB for the given passband, stopband, ripple, attenuation, and sampling frequency specifications.

- To determine the minimum filter order and corresponding cutoff frequencies and obtain the filter coefficients for both IIR filter designs.

- To analyze and compare the frequency-domain characteristics of the Butterworth and Chebyshev Type-I filters using magnitude response, phase response, and group delay plots.

- To evaluate the stability and performance of the designed filters using pole-zero plots, impulse responses, and pole magnitude analysis.

## Procedure

- Open MATLAB and create a new script file.

- Define the filter specifications, including sampling frequency, passband frequencies, stopband frequencies, passband ripple, and stopband attenuation.

- Normalize the passband and stopband frequencies with respect to the Nyquist frequency

(𝐹𝑠/2).

- Use the buttord() function to determine the minimum order and cutoff frequencies of the Butterworth Band-Pass Filter.

- Design the Butterworth Band-Pass Filter using the butter() function and obtain its numerator and denominator coefficients.

- Use the cheb1ord() function to determine the minimum order and cutoff frequencies of the Chebyshev Type-I Band-Pass Filter.

- Design the Chebyshev Type-I Band-Pass Filter using the cheby1() function and obtain its numerator and denominator coefficients.

- Display the filter order, cutoff frequencies, and filter coefficients for both filters.

- Plot and analyze the frequency responses of the Butterworth and Chebyshev Type-I filters using the freqz() function.

- Compare the magnitude responses and phase responses of both filters using frequency response plots.

- Generate pole-zero plots using the zplane() function and observe the pole and zero locations.

- Obtain and compare the group delay characteristics using the grpdelay() function.

- Plot the impulse responses of both filters using the impz() function.

- Determine the pole magnitudes using the roots() function and verify filter stability by checking whether all poles lie inside the unit circle.

- Analyze and compare the overall performance of the Butterworth and Chebyshev Type-I Band-Pass Filters based on their frequency responses, group delay, impulse response, and stability characteristics.

## Program (Matlab)

clc; clear; close all;

% Specifications Fs=2000; Rp=1; Rs=40; Wp=[300 600]/(Fs/2); Ws=[200 700]/(Fs/2);

% Butterworth Filter [n1,W1]=buttord(Wp,Ws,Rp,Rs);

[b1,a1]=butter(n1,W1,'bandpass');

% Chebyshev Type-I Filter [n2,W2]=cheb1ord(Wp,Ws,Rp,Rs); [b2,a2]=cheby1(n2,Rp,W2,'bandpass');

% Magnitude Response figure;

freqz(b1,a1); title('Butterworth Filter');

figure;

freqz(b2,a2); title('Chebyshev Type-I Filter');

% Group Delay figure;

grpdelay(b1,a1); hold on; grpdelay(b2,a2); legend('Butterworth','Chebyshev-I');

% Pole-Zero Plot figure;

subplot(121),zplane(b1,a1),title('Butterworth') subplot(122),zplane(b2,a2),title('Chebyshev-I')

% Impulse Response figure;

subplot(211),impz(b1,a1),title('Butterworth Impulse') subplot(212),impz(b2,a2),title('Chebyshev Impulse')

% Pole Magnitude figure;

subplot(121),stem(abs(roots(a1)),'filled') title('Butterworth Pole Magnitude')

subplot(122),stem(abs(roots(a2)),'filled') title('Chebyshev Pole Magnitude')

disp(['Butterworth Order = ',num2str(n1)]) disp(['Chebyshev Order = ',num2str(n2)])

## Output:

Butterworth Order = 9 Chebyshev Order = 5

## Results:

- Butterworth and Chebyshev Type-I IIR Band-Pass Filters were successfully designed and implemented in MATLAB.

- The minimum filter orders and cutoff frequencies were obtained for both filters.

- The frequency responses were analyzed and compared; the Chebyshev filter provided a sharper transition, while the Butterworth filter showed a smoother response.

- Pole-zero analysis confirmed that both filters are stable and satisfy the given design specifications.