| EX NO:1 | Butterworth IIR Filters |
| --- | --- |
| DATE: | 1.1. Design and Analysis of a Butterworth IIR Low-Pass Filter Using MATLAB. |

## Objectives

- To determine the minimum filter order and cutoff frequency, required to satisfy the design constraints.

- To design and analyze the magnitude response, phase response, characteristics of the Butterworth IIR low-pass filter for the given passband and stopband specifications.

- To analyze the impulse response, step response and group delay characteristics of the Butterworth IIR low-pass filter for the given passband and stopband specifications.

- To examine the pole-zero plot and verify the stability and performance of the designed low-pass filter.

## Procedure

- Open MATLAB and create a new script file in the MATLAB Editor.

- Define the filter specifications such as passband ripple (𝑅𝑝), stopband attenuation (𝑅𝑠), passband frequency (𝐹𝑝), stopband frequency (𝐹𝑠𝑏), and sampling frequency (𝐹𝑠).

- Calculate the normalized passband and stopband frequencies with respect to the Nyquist frequency (𝐹𝑠/2).

- Use the buttord() function to determine the minimum order (𝑁) and normalized cutoff frequency (𝑊𝑛) required for the Butterworth low-pass filter.

- Design the Digital IIR Butterworth Low-Pass Filter using the butter() function and obtain the numerator (𝑏) and denominator (𝑎) filter coefficients.

- Display the filter order and cutoff frequency obtained from the design process.

- Compute the frequency response of the designed filter using the freqz() function.

- Plot the magnitude response of the filter and observe the passband characteristics and stopband attenuation.

- Plot the phase response of the filter and analyze the phase variation over the frequency range.

- Generate the pole-zero plot using the zplane() function to verify the stability of the designed IIR filter.

- Create a test signal consisting of low-frequency and high-frequency components.

- Apply the designed IIR low-pass filter to the test signal using the filter() function.

## Program (Matlab Code)

clc; clear

Fs=10000; Fp=1000; Fst=1500;

[N,Wn]=buttord(Fp/(Fs/2),Fst/(Fs/2),1,60); [b,a]=butter(N,Wn);

freqz(b,a)	% Magnitude & Phase

figure [h,n]=impz(b,a,50);

…title('Impulse Response')

figure stepz(b,a)

figure zplane(b,a)

fprintf('Minimum Filter Order (N) = %d\n',N); fprintf('Cutoff Frequency (Wn) = %.4f\n',Wn); Input:

- Passband Ripple (𝑅𝑝) = 1 dB

- Stopband Attenuation (𝑅𝑠) = 20 dB

- Passband Frequency (𝐹𝑝) = 1000 Hz

- Stopband Frequency (𝐹𝑠) = 3000 Hz

- Sampling Frequency (𝐹𝑠𝑎𝑚𝑝𝑙𝑖𝑛𝑔) = 8000 Hz

- Filter Choice = Low-Pass Filter (LPF)

Output:

Objective 1:

Minimum Filter Order (N) = 17 Cutoff Frequency (Wn) = 0.2083

## Objective 2:

.

Objective 3:

Objective 4:

## Result:

- The minimum filter order and cutoff frequency satisfying the given specifications were determined successfully.

- The magnitude and phase responses of the Butterworth IIR low-pass filter were obtained and analyzed.

- The impulse response, step response, and group delay characteristics were evaluated successfully.

- The pole-zero plot confirmed the stability and proper performance of the designed filter.

| EX NO:1 | Butterworth IIR Filters |
| --- | --- |
| DATE: | 1.2. Design and Analysis of a Butterworth IIR High-Pass Filter Using MATLAB. |

## Objective

  - To determine the minimum filter order and cutoff frequency, required to satisfy the design constraints.

  - To design and analyze the magnitude response, phase response, characteristics of the Butterworth IIR High-pass filter for the given passband and stopband specifications.

  - To analyze the impulse response, step response and group delay characteristics of the Butterworth IIR High-pass filter for the given passband and stopband specifications.

  - To examine the pole-zero plot and verify the stability and performance of the designed High-pass filter.

## Procedure

- Open MATLAB and create a new script file.

- Enter the filter specifications: passband ripple (𝑅𝑝), stopband attenuation (𝑅𝑠), passband frequency (𝐹𝑝), stopband frequency (𝐹𝑠), and sampling frequency (𝐹𝑠𝑎𝑚𝑝𝑙𝑖𝑛𝑔).

- Normalize the passband and stopband frequencies with respect to the Nyquist frequency

(𝐹𝑠𝑎𝑚𝑝𝑙𝑖𝑛𝑔/2).

- Use the buttord() function to determine the minimum filter order and cutoff frequency for the Butterworth high-pass filter.

- Design the IIR Butterworth high-pass filter using the butter() function and obtain the numerator and denominator coefficients.

- Display the filter order and cutoff frequency.

- Compute the frequency response of the filter using the freqz() function.

- Plot the magnitude response and verify that low-frequency components are attenuated while high-frequency components are passed.

- Plot the phase response and analyze the phase characteristics.

- Generate the pole-zero plot using the zplane() function and verify that all poles lie inside the unit circle.

- Create a test signal consisting of low-frequency and high-frequency sinusoidal components.

- Apply the designed high-pass filter to the test signal using the filter() function.

- Plot and compare the input and filtered output signals.

- Analyze the filter performance in terms of frequency selectivity and stability.

- Verify that the designed filter satisfies the given specifications.

## Program (Matlab Code)

clc; clear

Fs=10000; Fp=3000; Fst=2000;

[N,Wn]=buttord(Fp/(Fs/2),Fst/(Fs/2),1,60);

[b,a]=butter(N,Wn,'high');

% Magnitude & Phase freqz(b,a)

% Impulse Response figure [h,n]=impz(b,a,50); stem(n,h), grid on title('Impulse Response')

% Step Response figure

stepz(b,a)

title('Step Response')

% Group Delay figure grpdelay(b,a) title('Group Delay')

% Pole-Zero Plot figure zplane(b,a)

title('Pole-Zero Plot')

fprintf('Minimum Filter Order (N) = %d\n',N); fprintf('Cutoff Frequency (Wn) = %.4f\n',Wn); Output:

## Objective 1:

Minimum Filter Order (N) = 12 Cutoff Frequency (Wn) = 0.5807

## Objective 2:

## Objective 3:

## Objective 4:

Result:

- The minimum filter order and cutoff frequency satisfying the given specifications were determined successfully.

- The magnitude and phase responses of the Butterworth IIR High-pass filter were obtained and analyzed.

- The impulse response, step response, and group delay characteristics were evaluated successfully.

- The pole-zero plot confirmed the stability and proper performance of the designed filter.